Supplementary Material

Expanding Digital Physics Laboratories for Student Inquiry with Generative AI: A Modular Framework

S1. Source Files and Resources

The source repository provides the reusable Raspberry Pi Pico W software templates, three implemented sensor configurations, and resources for adapting the framework to additional measurements.

Repository: https://github.com/klyangBUAS/A-Modular-Framework

The principal files are:

- main.py: MicroPython code for sensor acquisition and data processing.

- index.html: Browser-based interface for displaying measurements, plotting data, and exporting results.

- Sensor-specific programs for HC-SR04 distance measurement, HX711-based force measurement, and BH1750 light-intensity measurement.

- Wiring information, a reusable AI adaptation prompt, and a teacher verification checklist.

The shared templates provide the starting point for adaptation, while the sensor-specific programs illustrate how the framework can be applied to different physical quantities.

S2. Sensor Interfaces

The three sensor configurations use the following Pico W connections. They were tested separately rather than connected simultaneously.

| Sensor module | Measurement | Pico W connections |
| --- | --- | --- |
| HC-SR04 | Distance | Trigger: GP16; Echo: GP17 |
| HX711 | Force using a load cell | DOUT: GP16; SCK: GP17 |
| BH1750 | Light intensity | SDA: GP16; SCL: GP17 |

GP16 and GP17 correspond to physical header pins 21 and 22, respectively. The Pico W’s 3V3(OUT) and GND connections are physical pins 36 and 38. Before connecting a module, consult the repository’s wiring information and the specifications of the hardware in use. Ensure that sensor output voltages are compatible with the Pico W’s 3.3-V GPIO inputs.

S3. Reusable AI Adaptation Prompt

The following prompt can be adapted when extending the existing framework.

I am extending an existing Raspberry Pi Pico W digital physics experiment. Use the supplied main.py and index.html as the starting point rather than creating an unrelated program.

Sensor model: [model]

Physical quantity and unit: [quantity and unit]

Communication protocol and pin connections: [details]

Power supply and signal-voltage requirements: [details]

Measurement range and calibration or conversion method: [details]

Sampling requirements and desired graphs or data export: [details]

Student inquiry task: [question students will investigate]

Identify the necessary changes to sensor initialization, data acquisition, processing, calibration, and visualization. Preserve the existing shared functions and interface where possible. Provide the complete revised files and explain the changes, assumptions, units, and wiring. If essential hardware information is missing, ask for it rather than inventing specifications. Do not claim that the code has been tested on hardware unless such testing has actually been performed.

S4. Teacher Verification Checklist

Before using an adapted module with students, check the following:

- Connections: Wiring matches the code and sensor documentation.

- Electrical compatibility: Supply and signal voltages are appropriate for the Pico W.

- Data acquisition: The sensor returns interpretable readings.

- Physical quantity and units: Raw readings are converted appropriately, and displayed units are correct.

- Calibration: The calibration procedure is suitable for the intended measurement.

- Plausibility: Readings respond as expected when experimental conditions change.

- Graphs and data export: Labels, units, plots, and exported data are consistent.

- Classroom readiness: Known limitations and likely sources of error have been considered.

If a check fails, revise the code or hardware setup and repeat the relevant test. A functioning display or plausible response alone does not establish measurement accuracy; quantitative accuracy claims require comparison with an appropriate reference.